PROJECT 1

Anti-Sleep Arduino Car with OpenCV & Bluetooth

Eye Detection System using Python + HC-05 + L298N Motor Driver

# 1. Experiment

This project creates an anti-sleep safety system for a remote-controlled Arduino car. The car moves forward continuously as long as the driver is awake and looking at the laptop camera. When the driver falls asleep (eyes closed for 2+ seconds), the car stops immediately via Bluetooth communication.

The system uses OpenCV and MediaPipe library to detect the driver's eyes in real time through the laptop webcam. The Eye Aspect Ratio (EAR) algorithm measures how open or closed the eyes are. When EAR drops below the threshold for 2 seconds, Python sends a STOP command via Bluetooth to the Arduino, which controls the motors.

# 2. Requirements

## Software Requirements

- Python 3.11 or above

- Arduino IDE 2.3 or above

- VS Code (recommended editor)

- Python Libraries: opencv-python, mediapipe, pyserial

- Arduino Library: SoftwareSerial (built-in)

## Hardware Requirements

- Arduino Uno board

- HC-05 Bluetooth module

- L298N Motor Driver module

- 2x DC Motors (500 RPM)

- 9V or 12V Battery for motors

- Laptop with webcam

- Jumper wires

- Breadboard (optional)

# 3. Components

Component

Arduino Uno

HC-05 Bluetooth

L298N Motor Driver Car metal chassis

DC Motor 500RPM -4

18650 battery -2

Jumper wire kit

Laptop Camera

# 4. Connectivity / Wiring

## HC-05 Bluetooth Module to Arduino

| HC-05 Pin | Arduino Pin | Notes |
| --- | --- | --- |
| VCC | 5V | Power supply |
| GND | GND | Ground |
| TX | Pin 10 (RX) | HC-05 sends to Arduino |
| RX | Pin 11 (TX) | Arduino sends to HC-05 |

## L298N Motor Driver to Arduino

| L298N Pin | Arduino Pin | Notes |
| --- | --- | --- |
| IN1 | Pin 2 | Motor A direction 1 |
| IN2 | Pin 3 | Motor A direction 2 |
| IN3 | Pin 4 | Motor B direction 1 |
| IN4 | Pin 5 | Motor B direction 2 |
| ENA | Jumper ON | Motor A always full speed |
| ENB | Jumper ON | Motor B always full speed |
| GND | GND | Common ground |
| 12V | Vin | Motor power supply |

## Motors to L298N

| Motor | L298N Pins |
| --- | --- |
| Motor A (Left wheel) | OUT1 (Negative), OUT2 (Positive) |
| Motor B (Right wheel) | OUT3 (Positive), OUT4 (Negative) |

## Power Connection

| 18650 battery | L298N Pins |
| --- | --- |
| Motor A (Left wheel) | +12v |
| Motor B (Right wheel) | Gnd |

# 5. Steps Procedure

## Step 1 - Install Python Libraries

pip install mediapipe==0.10.9

pip install opencv-python pyserial

## Step 2 - Pair HC-05 with Laptop Bluetooth

- Power on Arduino with HC-05 connected (LED blinks fast)

- Windows 11: Settings > Bluetooth > Add Device > HC-05

- Enter PIN: 1234(connect fast)

- Open Device Manager > Ports > Note the COM port number (e.g. COM7)

## Step 3 - Upload Arduino Code

- Disconnect HC-05 TX and RX pins (Pin 10 and 11) from Arduino

- Open Arduino IDE, select Board: Arduino UNO and Port: COM3

- Upload the Arduino code provided below

- After upload complete, reconnect HC-05 TX to Pin 10 and RX to Pin 11

## Step 4 - Update Python Code

- Open python file in VS Code

- Change BLUETOOTH_PORT = 'COM7' to your actual COM port number

- Save the file

## Step 5 - Run the Project

- Power on Arduino

- Place car on the floor pointing forward

- Run Python: python carr_control.py(example file name)

- Sit in front of laptop camera

- Car moves forward when eyes are open

- Car stops when eyes close for 2+ seconds

# 6. Code

## Arduino Code (EYE_ANTISLEEP_CAR_OFF.ino)

#include <SoftwareSerial.h>

SoftwareSerial BT(10, 11); // RX, TX

int IN1 = 2, IN2 = 3;

int IN3 = 4, IN4 = 5;

void setup() {

pinMode(IN1, OUTPUT); pinMode(IN2, OUTPUT);

pinMode(IN3, OUTPUT); pinMode(IN4, OUTPUT);

BT.begin(9600);

Serial.begin(9600);

stopCar();

}

void loop() {

if (BT.available()) {

char cmd = BT.read();

if (cmd == 'F') { moveForward(); }

else if (cmd == 'S') { stopCar(); }

}

}

void moveForward() {

digitalWrite(IN1, HIGH); digitalWrite(IN2, LOW);

digitalWrite(IN3, HIGH); digitalWrite(IN4, LOW);

}

void stopCar() {

digitalWrite(IN1, LOW); digitalWrite(IN2, LOW);

digitalWrite(IN3, LOW); digitalWrite(IN4, LOW);

}

## Python Code (carr_control.py)

import cv2

import mediapipe as mp

import serial

import time

BLUETOOTH_PORT = 'COM7'

BAUD_RATE      = 9600

EAR_THRESHOLD  = 0.25

CLOSED_SECONDS = 2.0

mp_face_mesh = mp.solutions.face_mesh

face_mesh = mp_face_mesh.FaceMesh(

max_num_faces=1, refine_landmarks=True,

min_detection_confidence=0.5,

min_tracking_confidence=0.5

)

LEFT_EYE  = [362, 385, 387, 263, 373, 380]

RIGHT_EYE = [33, 160, 158, 133, 153, 144]

def eye_aspect_ratio(landmarks, eye_points, w, h):

points = []

for i in eye_points:

x = int(landmarks[i].x * w)

y = int(landmarks[i].y * h)

points.append((x, y))

A = ((points[1][0]-points[5][0])**2+(points[1][1]-points[5][1])**2)**0.5

B = ((points[2][0]-points[4][0])**2+(points[2][1]-points[4][1])**2)**0.5

C = ((points[0][0]-points[3][0])**2+(points[0][1]-points[3][1])**2)**0.5

return (A + B) / (2.0 * C)

bt = serial.Serial(BLUETOOTH_PORT, BAUD_RATE, timeout=1)

time.sleep(2)

cap = cv2.VideoCapture(0)

car_moving = False

eyes_closed_start = None

while True:

ret, frame = cap.read()

if not ret: break

h, w = frame.shape[:2]

rgb = cv2.cvtColor(frame, cv2.COLOR_BGR2RGB)

results = face_mesh.process(rgb)

eyes_open = True

if results.multi_face_landmarks:

landmarks = results.multi_face_landmarks[0].landmark

left_ear  = eye_aspect_ratio(landmarks, LEFT_EYE, w, h)

right_ear = eye_aspect_ratio(landmarks, RIGHT_EYE, w, h)

ear = (left_ear + right_ear) / 2.0

if ear < EAR_THRESHOLD:

eyes_open = False

if eyes_closed_start is None:

eyes_closed_start = time.time()

closed_duration = time.time() - eyes_closed_start

if closed_duration >= CLOSED_SECONDS and car_moving:

bt.write(b'S')

car_moving = False

else:

eyes_closed_start = None

if eyes_open and not car_moving:

bt.write(b'F')

car_moving = True

else:

if car_moving:

bt.write(b'S')

car_moving = False

cv2.imshow('Anti-Sleep Car', frame)

if cv2.waitKey(1) & 0xFF == ord('q'): break

bt.write(b'S')

cap.release()

cv2.destroyAllWindows()

bt.close()

# 7. Where to Buy (India)

| Component | Website | Price |
| --- | --- | --- |
| Arduino Uno | https://robocraze.com/products/uno-smd-board-compatible-with-arduino | 229 |
| HC-05 Bluetooth | https://robocraze.com/products/hc-05-bluetooth-module | 224 |
| L298N Motor Driver | https://robocraze.com/products/l298-motor-driver-module | 139 |
| 500 RPM DC Motor | https://robocraze.com/products/500-rpm-geared-motor | 155 |
| Jumper Wires kit (enough for all projects) | https://robocraze.com/products/jumper-wire-set-m2m-m2f-f2f-40-pcs-each | 128 |
| Car metal chassis | https://amzn.in/d/03Hg6WWs | 309 |
| 18650 batteries -2 | https://robocraze.com/products/3-7v-1200mah-18650-battery | 40 |
| Battery case | https://robocraze.com/products/18650-2-cell-holder | 21 |
| wheels | https://robocraze.com/products/bo-motor-wheels-4-pcs | 81 |
| USB cable | https://robocraze.com/products/usb-a-b-cable-3-0-cm | 33 |
| On off switch | https://robocraze.com/products/2-pin-mini-on-off-spst-rocker-switch-19-x-13mm | 25 |
|  | total | 1384˜1450 |

Circuit diagram